MATERIAL SAFETY DATA SHEET. 	SAFCSEAL.DOC 1 of 2

1.	IDENTIFICATION OF PRODUCT AND COMPANY

Product name : 		CRETA-SEAL

Water based concrete sealer, water- & dust-proofer

Company        :  D. Adam & Associates / ACTEL Coatings

Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ Scotland, U.K.

Tel: +44  (0) 7880 805478  E:  dougadam@globalnet.co.uk    www.actel.f9.co.uk

2.	COMPOSITION / HAZARDOUS INGREDIENTS

Ingredient name		Cas No.		Contents	Classification

Sodium silicate		1344-09-8	 35-40%

3.	HAZARD IDENTIFICATION

On available data this preparation does not carry a hazard classification according to EEC 	directives 88/379/EEC and 79/831/EEC.

This product may cause soreness and irritation to skin and eyes. Harmful if swallowed. Wash 	after handling.

4.	FIRST AID MEASURES

Eye contact :	Wash out with copious amounts of water.

If irritation persists seek medical advice.

Skin contact :	Wash off with soap and water for 15 minutes. Seek medical attention.

Ingestion:	Give water, milk or milk of magnesia to drink. Obtain medical advice. Do not 			induce vomiting.

Inhalation:	Move exposed person to fresh air. If breathing is affected give oxygen. Keep 			warm. Seek medical attention.

5.	FIRE FIGHTING MEASURES

This preparation is stable and non-combustible. Use extinguishing media for surrounding fire.

6.	ACCIDENTAL RELEASE MEASURES

Spillages are considered harmless, however they should be contained using soil or other 	adsorbent material to prevent contamination of drains and water courses. Waste material

from spillages should be disposed of in accordance to local regulations. Flush spill area with 	water. Spilled material may cause surfaces to become slippery.

7.	HANDLING AND STORAGE

Store securely in the original labelled containers. Do not store above 50oC for prolonged 	periods.

SAFCSEAL.DOC 2 of 2

8.	EXPOSURE CONTROLS / PERSONAL PROTECTION

Exposure controls : 	Respiratory protection is not normally required. Ensure adequate 				ventilation in the work area.

Personal protection :	Respiratory, wear a simple mask to prevent inhalation of liquid

as a vapour if ventilation is inadequate.

Hands, the use of suitable gloves is recommended.

Eyes, safety glasses or goggles, (BS2902).

Skin. Apron, overalls

9.	PHYSICAL AND CHEMICAL PROPERTIES

State			: Colourless liquid

Colour			: Water clear

pH-value		: >11

Boiling point		: 100oC

Flash point		: N/A

Solubility		: Water miscible in all proportions

Density @ 25oC	: 1.2 - 1.6

Odour			: Characteristic slightly soapy.

10.	STABILITY AND REACTIVITY

This product is stable in excess of  one year when stored between 5OC and 30OC. Hazardous 	polymerisation will not occur.

Incompatibles: Mineral acids, organic acids most common metals, organic materials

11.	TOXICOLOGICAL INFORMATION

This preparation does not carry a hazard classification.

12.	ECOLOGICAL INFORMATION

No data available. Do not release into surface waters.

13.	DISPOSAL CONSIDERATIONS

The product is not classified as hazardous waste.

Disposal of large quantities as waste must always comply with existing EEC,  national and

local regulations. Registered waste carriers and licensed disposal sites must be used.

14.	Transport considerations

Not classified as hazardous for transportation according to international regulations. Proper 	shipping name: Chemicals n.o.s. (non regulated)

15.	REGULATORY INFORMATION

This preparation does not carry a hazard classification according to EEC directives

88/379/EEC, 67/548/EEC and 79/831/EEC.

16.	OTHER INFORMATION

This Material Safety Data Sheet follows the requirements of EEC directive 91/155/EEC.

Revision date May 99