MATERIAL SAFETY DATA SHEET. 		SAFEFLY.DOC page 1 of 2

1.	IDENTIFICATION OF PRODUCT AND COMPANY

Product name :   	FLY AWAY.

Non stick coating to prevent fly posting.

Company        :  D. Adam & Associates / ACTEL Coatings

Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ Scotland, U.K.

Tel: +44  (0) 7880 805478  E:  dougadam@globalnet.co.uk    www.actel.f9.co.uk

2.	COMPOSITION/ HAZARDOUS INGREDIENTS

Ingredient name				Cas No.		Contents	Classification    Risk

WHITE SPIRIT				64742-82-1	<60%				R10

3.	HAZARD IDENTIFICATION

On available data this preparation does not carry a hazard classification according to EEC 	directives 88/379/EEC and 79/831/EEC.

This product is flammable flash point  38 oC

This product may cause soreness in contact with eyes.

Prolonged contact with skin may result in irritation.

4.	FIRST AID MEASURES

Eye contact :	Wash out with copious amounts of water.

If irritation persists seek medical advice.

Skin contact :	Wash off with soap and water.

Ingestion      :  Wash out the mouth with water. Obtain medical advice.

Inhalation     :  Move the exposed person into fresh air and get prompt medical attention.

5.	FIRE FIGHTING MEASURES

This preparation is flammable and will burn if ignited. Use extinguishing media appropriate 	for solvent fires.

6.	ACCIDENTAL RELEASE MEASURES

Spillages should be contained using soil or other adsorbent material to prevent 	contamination of 	drains and water courses. Waste material from spillages should be disposed of according to local 	regulations. Spilled material may cause surfaces to become slippery. Clean-up personnel should 	use respiratory and/or liquid contact protection.

7.	HANDLING AND STORAGE

Store securely in the original labelled containers out of reach of children.

Keep in cool, dry well ventilated conditions away from heat sparks and open flames.

8.	EXPOSURE CONTROLS / PERSONAL PROTECTION

Ingredient name			Cas No.		Std.Lt. Exp. (8hrs)	St. Exp.(10 min)

White spirit			64742-82-1	100 ppm		125 ppm

Exposure controls : 	Respiratory protection is not normally required but if spraying

applications are involved appropriate masks should be worn.

SAFEFLY.DOC page 2 of 2

8. EXPOSURE CONTROLS / PERSONAL PROTECTION, Contd.

Exposure controls   :  Ensure sufficient local exhaust ventilation

Personal protection :	Respiratory, wear a gas mask filter A, to prevent inhalation of liquid

as a vapour.

Hands, the use of suitable gloves is recommended.

Eyes, safety glasses or goggles, (BS2902).

Skin. Apron, overalls

9.	PHYSICAL AND CHEMICAL PROPERTIES

State			:  Thick Liquid.		Flash point		:  38 oC

Colour			:  Opaque white.	Solubility		:  Not water miscible		pH-value		:  N/A			Density @ 25oC	:  0.85

Boiling point		:  oC 			Odour			:  Solvent

10.	STABILITY AND REACTIVITY

This product is stable in excess of  one year when stored between 5OC and 30OC.

11.	TOXICOLOGICAL INFORMATION

This preparation does not carry a hazard classification.

12.	ECOLOGICAL INFORMATION

No data available.  Do not release into surface waters.

13.	DISPOSAL CONSIDERATIONS

The product is not classified as hazardous waste.

Disposal of large quantities as waste must always comply with existing EEC,  national and

local regulations. Registered waste carriers and licensed disposal sites must be used.

14.	Transport considerations

ADR/RID	:  Class :  3.41		   Item  :	Name  :   Petroleum distillate

IMDG		:  Class :  3.3		UN-nr.  : 1268

Proper shipping name		: Petroleum distillate NOS     Label :  3

Marine pollutant		: ---/No		EMS	            MFG

ICAO/IATA	:  Class  :  3		UN-nr.   : 1268

Proper shipping name		 : Petroleum distillate NOS     Label :  RFL

15.	REGULATORY INFORMATION

15.1	Classification	:   Petroleum distillate,  flammable

15.1	Labelling	:   Flammable liquid

15.2	Symbol (s)	:

15.3	R-phrases	:   R - 10,  Flammable

15.4	S-phrases	:   S - 16,  Keep away from sources of ignition. No smoking.

S - 25,  Avoid contact with eyes.

S - 26,  In case of contact with eyes, rinse immediately with plenty of 					    water and seek medical advice.

16.	OTHER INFORMATION

This Material Safety Data Sheet follows the requirements of EEC directive 91/155/EEC.

Revised  May 1999