MATERIAL SAFETY DATA SHEET. 	SAFEGTI.DOC Page 1 of 3

1.	IDENTIFICATION OF PRODUCT AND COMPANY

Product name : 	CLEANAWAY GTi.

Heavy duty water miscible graffitti remover for porous surfaces

Company        :  D. Adam & Associates / ACTEL Coatings

Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ Scotland, U.K.

Tel: +44  (0) 7880 805478  E:  dougadam@globalnet.co.uk    www.actel.f9.co.uk

2.	COMPOSITION/ HAZARDOUS INGREDIENTS

Ingredient name				Cas No.		Contents	Classification

N-Methyl-2-pyrolidone			872-50-4	>60%		Xi,  R36/38. S41.

3.	HAZARD IDENTIFICATION

On available data this preparation has a hazard classification as an Irritant according to 	EEC directives 88/379/EEC and 79/831/EEC.

Identification of risk and safety phrases

R36/38		Irritating to eyes and skin.

R20		Harmful by inhalation.

R36/37/38	Irritating to eyes, respiratory system and skin.

S41		In case of fire and/or explosion do not breathe fumes.

4.	FIRST AID MEASURES

Eye contact :	Wash out with copious amounts of water.

If irritation persists seek medical advice.

Skin contact :	Wash off with soap and water.

Ingestion      :  Wash out the mouth with water. Obtain medical advice.

Inhalation     :  Move the exposed person into fresh air at once.

Get prompt medical attention.

5.	FIRE FIGHTING MEASURES

This preparation is combustible and will burn if ignited. Use extinguishing media appropriate 	for surrounding fire. Hazardous decomposition products from combustion, very toxic 	gases/vapours/fumes of carbon monoxide (CO), oxides of nitrogen.

6.	ACCIDENTAL RELEASE MEASURES

Spillages should be contained using soil or other adsorbent material to prevent 	contamination of drains and water courses. Waste material from spillages should be 	disposed of in accordance to local regulations. Spilled material may cause surfaces to 	become slippery. Clean-up personnel should use respiratory and/or liquid contact protection.

SAFEGTI.DOC  Page 2 of 3

7.	HANDLING AND STORAGE

Store securely in the original labelled containers out of reach of children.

Keep in cool, dry well ventilated conditions away from heat sparks and open flames.

8.	EXPOSURE CONTROLS / PERSONAL PROTECTION

Ingredient name			Cas No.		Std.Lt. Exp. (8hrs)	St. Exp.(10 min)

N-Methyl-2-pyrolidone		872-50-4	OES   100 ppm		No std.

Exposure controls : 	Respiratory protection is not normally required but if spraying 					applications are involved appropriate masks should be worn.

Personal protection :	Respiratory, wear a simple mask to prevent inhalation of liquid

as a vapour.

Hands, the use of suitable gloves is recommended.

Eyes, safety glasses or goggles, (BS2902).

Skin. Apron, overalls

9.	PHYSICAL AND CHEMICAL PROPERTIES

State			:  Liquid, hygroscopic.	Flash point		:  86 oC

Colour			:  Clear brown.		Solubility		:  Water miscible

pH-value		:  8.0			Density @ 25oC	:  1.03

Boiling point		:  oC 			Odour			:  Slight

10.	STABILITY AND REACTIVITY

This product is stable in excess of  one year when stored between 5OC and 30OC.

11.	TOXICOLOGICAL INFORMATION

This preparation does not carry a hazard classification.

12.	ECOLOGICAL INFORMATION

No data available. Do not release into surface waters.

13.	DISPOSAL CONSIDERATIONS

The product is not classified as hazardous waste.

Disposal of large quantities as waste must always comply with existing EEC,  national and

local regulations. Registered waste carriers and licensed disposal sites must be used.

14.	Transport considerations

Not classified as hazardous for transportation according to international regulations.

SAFEGTI.DOC  Page 3 of 3

15.	REGULATORY INFORMATION

Label for supply :		IRRITANT

Risk phrases :	R-20		Harmful by inhalation.

R-36/38		Irritating to the eyes and skin.

R36/37/38	Irritating to eyes, respiratory system and skin.

Safety phrases: S41		In case of fire and/or explosion do not breathe fumes.

16.	OTHER INFORMATION

This Material Safety Data Sheet follows the requirements of EEC directive 91/155/EEC.

Revised May 1999