MATERIAL SAFETY DATA SHEET. SAFESFREE.DOC Page 1 of 2

1.	IDENTIFICATION OF PRODUCT AND COMPANY

Product name : 	HYDRAWAX water displacing ultra thin rust preventative

Company        :  D. Adam & Associates  /  ACTEL Coatings,

# Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ  Scotland

Tell: +44 07780 805478

http://www.actel.f9.co.uk   E-Mail:  dougadam@globalnet.co.uk.

2.	COMPOSITION/ HAZARDOUS INGREDIENTS

Ingredient name				Cas No.		Contents	Classification    Risk

Oxidised petroleum oils			64742-99-0	80-85%		None

Petroleum barium sulphonate		61790-48-5	20-15%		Xn

3.	HAZARD IDENTIFICATION

Harmful if swallowed and by inhalation. Prolonged or repeated skin contact may cause

irritation to skin.

4.	FIRST AID MEASURES

Eye contact :	Wash thoroughly with water. If irritation persists seek medical advice.

Skin contact :	Wash off with soap and water. Remove grossly soiled clothing and launder

before reuse. Get medical attention if irritation results.

Ingestion      :  Obtain medical attention. Do not induce vomiting.

5.	FIRE FIGHTING MEASURES

Carbon Dioxide, foam, dry chemical and water fog media. Avoid breathing fire vapours.

Avoid water in straight stream; will spread fire.

6.	ACCIDENTAL RELEASE MEASURES

Spillages should be notified to Area authorities as appropriate and contained using soil or

other adsorbent material to prevent contamination of drains and water courses. Waste

material from spillages should be disposed of in accordance with local regulations. Spilled

material may cause surfaces to become slippery.

7.	HANDLING AND STORAGE

Store securely in the original labelled containers in cool, dry, well ventilated conditions away 	from heat sparks and open flames. No special storage precautions required.

8.	EXPOSURE CONTROLS / PERSONAL PROTECTION

Exposure controls : 	Respiratory protection is not normally required but if spraying 					applications are involved, air concentrations may require appropriate

masks to be worn.

Ensure sufficient local exhaust ventilation over heating processes.

Personal protection :	Respiratory, No special requirements under normal operations

Hands, use nitrile or butyl rubber gloves for prolonged or repeated

handling.

Eyes, chemical liquids safety glasses or goggles, (BS2902).

Skin. Gloves, overalls and good personal hygiene practices should

always be followed

SAFSFREE.DOC Page 2 of 2

9.	PHYSICAL AND CHEMICAL PROPERTIES

State			:  Wax solid		Flash point	:  140 oC

Colour			:  Brown		Solubility	:  Not water miscible

pH-value		:  N/A			Density @ 25oC: 0.95

Melt point		:  35oC 			Odour		:  Characteristic

10.	STABILITY AND REACTIVITY

This product is normally stable. Avoid extreme heat, strong oxidisers. Combustion of bulk

material may produce oxides of carbon, sulphur, barium and organic acids.

11. 	TOXICOLOGICAL INFORMATION

Eyes: Spray and vapour in the eyes may cause irritation and smarting.

Skin: Prolonged or repeated contact leads to drying and may defat the skin, but material is

not an irritant

Ingestion: May cause discomfort if swallowed

Inhalation: At normal ambient temperatures, product will be unlikely to present an inhalation hazard because of its low volatility. May be harmful by inhalation if exposure to vapours, mist or fumes resulting from thermal decomposition products occurs.

12.	ECOLOGICAL INFORMATION

Mobility: Spillages are unlikely to penetrate the soil

Persistence and Degradability: Not readily biodegradable

Aquatic toxicity: Spills may form films on water surfaces causing physical damage to organisms. Oxygen transfer could also be impaired.

Water endangering classification = WGK 1

13.	DISPOSAL CONSIDERATIONS

The product is not classified as hazardous waste.

Disposal of large quantities as waste must always comply with existing EEC,  national and

local regulations. Registered waste carriers and licensed disposal sites must be used.

14.	Transport considerations

Not classed as hazardous for transport (ADR, RID, UN, IMO, IATA/ICAO

15.	REGULATORY INFORMATION

15.1	Labelling	:   Xn

15.2	R-phrases	:   R20/22 Harmful by inhalation and if swallowed

15.3	S-phrases	:   S-24/25  Avoid contact with skin and eyes

S-28 After contact with skin, wash immediately with plenty of soap and

Water

S-36/37/39 Wear suitable protective clothing, gloves, and eye/face

protection

16.	OTHER INFORMATION

This MSDS dated May 2003 follows the requirements of EEC dangerous preparations

directives 91/155/EEC and 88/379

U.K. Regulatory references: Classification packaging and labelling regs. 1984 & 1994. Health

& Safety at Work Act 9174.CHIP Regs 1993. COSSH Regs 1988