MATERIAL SAFETY DATA SHEET. SAFEOUTL..DOC Page 1 of 3

1.	IDENTIFICATION OF PRODUCT AND COMPANY

Product name : 	OUTLINE

Water based graffitti “ghost” remover.

Company        :  D. Adam & Associates / ACTEL Coatings

Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ Scotland, U.K.

Tel: +44  (0) 7880 805478  E:  dougadam@globalnet.co.uk    www.actel.f9.co.uk

2.	COMPOSITION/ HAZARDOUS INGREDIENTS

Ingredient name				Cas No.		Contents	Classification

Sodium hydroxide			1310-58-3	<0.5%

Sodium hypochlorite			7681-52-9	<8%		Xi, R31-36/38

S2-28-50

3.	HAZARD IDENTIFICATION

On available data this preparation has a hazard classification as an Irritant according to 	EEC directives 88/379/EEC and 79/831/EEC.

Identification of risk and safety phrases

R36/38		Irritating to eyes and skin..

R31		Contact with acid libreates toxic gas.

S2		Keep out of reach of children.

S28		After contact with skin , wash immediately with plenty of water.

S50		Do not mix with acid based products or with any other cleaning chemicals.

4.	FIRST AID MEASURES

Eye contact :	Wash out with copious amounts of water.

If irritation persists seek medical advice.

Skin contact :	Wash off with soap and water.

Ingestion      :  Wash out the mouth with water. Obtain medical advice.

Inhalation     :  Move the exposed person into fresh air at once.

Get prompt medical attention.

5.	FIRE FIGHTING MEASURES

This preparation is non combustible. Use extinguishing media appropriate for surrounding 	fire.

6.	ACCIDENTAL RELEASE MEASURES

Spillages should be contained using soil or other adsorbent material to prevent 	contamination of drains and water courses. Waste material from spillages should be 	disposed of in accordance to local regulations. Spilled material may cause surfaces to 	become slippery. Clean-up personnel should use respiratory and/or liquid contact protection.

SAFEOUTL.DOC Page 2 of 3

7.	HANDLING AND STORAGE

Store securely in the original labelled containers out of reach of children.

8.	EXPOSURE CONTROLS / PERSONAL PROTECTION

Ingredient name			Cas No.		Std.Lt. Exp. (8hrs)	St. Exp.(10 min)

Sodium hydroxide		1310-58-3	     1 mg / m3   		     3 mg / m3

Sodium hypochlorite		7681-52-9	   25 p.p.m.			-

Exposure controls : 	Respiratory protection is not normally required but if spraying 					applications are involved appropriate masks should be worn.

Personal protection :	Respiratory, wear a simple mask to prevent inhalation of liquid

as a vapour.

Hands, the use of suitable gloves is recommended.

Eyes, safety glasses or goggles, (BS2902).

Skin. Apron, overalls

9.	PHYSICAL AND CHEMICAL PROPERTIES

State			:  Liquid.		Flash point		:  N/A

Colour			:  Clear straw.		Solubility		:  Water miscible	pH-value		:  13.0			Density @ 25oC	:  1.1

Boiling point		:   100 oC 		Odour			:  Slight

10.	STABILITY AND REACTIVITY

This product is stable in excess of  one year when stored between 5OC and 30OC.

11.	TOXICOLOGICAL INFORMATION

This preparation does not carry a hazard classification.

12.	ECOLOGICAL INFORMATION

No data available. Do not release into surface waters.

13.	DISPOSAL CONSIDERATIONS

The product is not classified as hazardous waste.

Disposal of large quantities as waste must always comply with existing EEC,  national and

local regulations. Registered waste carriers and licensed disposal sites must be used.

14.	Transport considerations

Not classified as hazardous for transportation according to international regulations.

SAFEOUTL.DOC Page 3of3

15. 	REGULATORY INFORMATION

Label for supply :		IRRITANT

Risk phrases :	R-31		Contact with acid liberates toxic gas.

R-36/38		Irritating to the eyes and skin.

Safety phrases:	S-2 		Keep out of reach of children.

S-28		After contact with skin, wash immediately with plenty of

clean water.

S-50		Do not mix with acid based products or other chemical 						cleaners.

16.	OTHER INFORMATION

This Material Safety Data Sheet follows the requirements of EEC directive 91/155/EEC.

Revised  May 1999