MATERIAL SAFETY DATA SHEET. SAFEPRF.DOC Page 1 of 2

1.	IDENTIFICATION OF PRODUCT AND COMPANY

Product name : 	STEEL PROOF.  Semi hard corrosion preventive.

Company        :  D. Adam & Associates  /ACTEL Coatings,

# Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ  Scotland

Tell: +44 07780 805478

www.actel.f9.co.uk   E-Mail:  dougadam@globalnet.co.uk.

2.	COMPOSITION/ HAZARDOUS INGREDIENTS

Ingredient name				Cas No.		Contents	Classification

Calcium soap of Oxidised wax		68476-81-3	60-50%		R36

Aliphatic hydrocarbon solvent		64742-48-9	40-50%		Xn, R10 R65

3.	HAZARD IDENTIFICATION

This product is flammable flash point  43 oC

This product may cause soreness in contact with eyes.

Prolonged contact with skin may result in irritation, dryness and cracking.

4.	FIRST AID MEASURES

Eye contact :	Wash out with copious amounts of water.

If irritation persists seek medical advice.

Skin contact :	Wash off with soap and water. Remove soiled clothing.

Ingestion      :  Do not induce vomiting. Drink plenty of water. Obtain medical advice if any

discomfort continues.

Inhalation     :  Move to fresh air. Keep warm. Get medical attention if discomfort continues.

5.	FIRE FIGHTING MEASURES

Water spray (a jet may scatter and spread fire) fog or mist, Powder, foam or CO2. Bulk

product in a fire situation, may form explosive or toxic mixtures with air.

6.	ACCIDENTAL RELEASE MEASURES

Spillages should be contained using soil or other adsorbent material to prevent 	contamination of drains and water courses. Waste material from spillages should be 	disposed of in accordance to local regulations. Spilled material may cause surfaces to 	become slippery. Clean-up personnel should use respiratory and/or liquid contact protection.

7.	HANDLING AND STORAGE

Store securely in the original labelled containers in cool, dry, well ventilated conditions away 	from oxidising agents, heat sparks and open flames. Avoid spilling, skin or eye contact.

8.	EXPOSURE CONTROLS / PERSONAL PROTECTION

Ingredient name			Cas No.		Std.	   Std.Lt. Exp. (8hrs

Aliphatic hydrocarbon solvent    64742-48-9	OEL		1000 mg/cu.m

Exposure controls : 	Respiratory protection is not normally required but if spraying 					applications are involved appropriate masks should be worn. 				             Ensure sufficient local exhaust ventilation.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION Contd.		 SAFESPRFR.DOC Page 2 of 2

Personal protection :	Respiratory, wear a gas mask filter A, to prevent inhalation of liquid

as a vapour and where OEL is reached.

Hands, the use of suitable gloves is recommended.

Eyes, safety glasses or goggles, (BS2902).

Skin. Apron, overalls

Observe good personal hygiene practices

9.	PHYSICAL AND CHEMICAL PROPERTIES

State			:  Brown soft solid	Flash point	:  43 oC

Colour			:  Brown.		Solubility	:  Not water miscible

pH-value		:  N/A			Density @ 25oC:  0.87

Boiling point		:  oC 			Odour		:  Characteristic

10.	STABILITY AND REACTIVITY

This product is stable, but avoid heat sparks and flames and strong oxidisers.

Fire creates toxic gases of Carbon Monoxide and Carbon Dioxide when bulk product burns

11.	TOXICOLOGICAL INFORMATION

May irritate eyes, respiratory system and skin. Take measures to wear protection and

maintain good personal hygiene practices.

12.	ECOLOGICAL INFORMATION

Spillages are unlikely to penetrate the soil.

Do not release into surface waters. Not readily biodegradable. Spills may form a film on

water causing damage to organisms. Oxygen transfer could also be impaired.

Water endangering classification is WGK 1

13.	DISPOSAL CONSIDERATIONS

Try to recycle product if possible

Disposal of large quantities as waste must always comply with existing EEC,  national and

local regulations. Registered waste carriers and licensed disposal sites must be used.

14.	Transport considerations

Not classified as hazardous for transport under notice no. 669 – carriage of goods by road

and rail classification, packaging and labelling regulations.

15.	REGULATORY INFORMATION

15.1	Classification	:   Petroleum distillate,  flammable

15.2	Labelling	:   Irritant

15.3	R-phrases	:   R - 10,  Flammable R -36 Irritating to eyes R-66 Repeated exposure may

cause skin dryness or cracking

15.4	S-phrases	:   S - 25,  Avoid contact with eyes.

S - 26,  In case of contact with eyes, rinse immediately with plenty of 					    water and seek medical advice.

S – 37  Wear suitable gloves

S – 51 Use only in well ventilated areas

S – 60 This material must be disposed of as hazardous waste

16.	OTHER INFORMATION

This Material Safety Data Sheet dated May 1999 follows the requirements of EEC directive 	91/155/EEC

Revised May 2003