MATERIAL SAFETY DATA SHEET. SAFEPRF.DOC Page 1 of 2

1.	IDENTIFICATION OF PRODUCT AND COMPANY

Product name : 	STEEL PROOF TFH  Thin film hard corrosion preventative.

Company        :  D. Adam & Associates  /ACTEL Coatings,

# Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ  Scotland

Tell: +44 07780 805478

http://www.actel.f9.co.uk   E-Mail:  dougadam@globalnet.co.uk.

2.	COMPOSITION/ HAZARDOUS INGREDIENTS

Ingredient name				Cas No.

Calcium soap of Oxidised wax		68476-81-3

Aliphatic hydrocarbon solvent		64742-48-9

3.	HAZARD IDENTIFICATION

This product contains an aliphatic volatile solvent

Contact with solvent vapour may cause irritation, headaches and dizziness

4.	FIRST AID MEASURES

Eye contact :	Wash out with copious amounts of water.

If irritation persists seek medical advice.

Skin contact :	Wash off with soap and water. Remove soiled clothing.

Ingestion      :  Do not induce vomiting. Obtain prompt medical attention. Keep at rest.

Inhalation     :  Move to fresh air. Keep warm. Get medical attention if discomfort continues.

5.	FIRE FIGHTING MEASURES

Carbon Dioxide, Powder, foam dry chemical or water fog. May generate irritating vapours

when burning. Wear breathing apparatus when fighting fire in enclosed environment.

NFPA ID: Health: 1,  Flammability: 1, Reactivity: 0

6.	ACCIDENTAL RELEASE MEASURES

Spillages should be contained using soil or other adsorbent material to prevent 	contamination of drains and water courses. Waste material from spillages should be 	disposed of in accordance to local regulations. Spilled material may cause surfaces to 	become slippery. Clean-up personnel should use respiratory and/or liquid contact protection.

7.	HANDLING AND STORAGE

Store securely in the original labelled containers in cool, dry, well ventilated conditions away 	from oxidising agents, heat sparks and open flames. Avoid spilling, skin or eye contact.

8.	EXPOSURE CONTROLS / PERSONAL PROTECTION

Ingredient name			Cas No.		Std.	   Std.Lt. Exp. (8hrs)

Aliphatic hydrocarbon solvent    64742-48-9	OEL	   1000 mg/cu.m

Exposure controls : 	Respiratory protection is not normally required but if spraying 					applications are involved appropriate masks should be worn. 				             Ensure sufficient local exhaust ventilation. A TLV of 100ppm is

suggested for solvent emissions.

8. EXPOSURE CONTROLS / PERSONAL PROTECTION Contd.		 SAFESPRFR.DOC Page 2 of 2

Personal protection :	Respiratory, No special requirements under normal conditions and

with adequate ventilation.

Hands, the use of suitable gloves is recommended.

Eyes, safety glasses or goggles, (BS2902).

Skin. Apron, overalls

Observe good personal hygiene practices

9.	PHYSICAL AND CHEMICAL PROPERTIES

State			:  Viscous liquid		Flash point	:  43 oC

Colour			:  Brown.		Solubility	:  Not water miscible

pH-value		:  N/A			Density @ 25oC:  0.90

Boiling point		:  173oC 		Odour		:  Characteristic

10.	STABILITY AND REACTIVITY

This product is stable, but avoid extreme heat and strong oxidisers.

Decomposes with heat. Hazardous polymerisation will not occur.

11.	TOXICOLOGICAL INFORMATION

Practically non toxic to skin and oral ingestion. Inhalation hazard negligible at ambient

temperatures –18oC to 25oC. High vapour concentrations may irritate eyes, respiratory

system and skin. Take measures to wear protection and maintain good personal hygiene

practices.

12.	ECOLOGICAL INFORMATION

Not established

13.	DISPOSAL CONSIDERATIONS

Try to recycle product if possible. Suitable for disposal by incineration.

Disposal of large quantities as waste must always comply with existing EEC,  national and

local regulations. Registered waste carriers and licensed disposal sites must be used.

14.	Transport considerations

Not classified as hazardous for transport under notice no. 669 – carriage of goods by road

and rail classification, packaging and labelling regulations.

15.	REGULATORY INFORMATION

15.1	Classification	:   N/A

15.2	Labelling	:   N/A

15.3	R-phrases	:   N/A

16.	OTHER INFORMATION

This Material Safety Data Sheet dated May 1999 follows the requirements of EEC directive 	91/155/EEC

Revised July 2003