MATERIAL SAFETY DATA SHEET. SAFGLZAA.DOC Page 1 of 2

1.	IDENTIFICATION OF PRODUCT AND COMPANY

Product name : 	ARMAGLAZE  PACK A of an Architectural / anti-graffitti coating

Company        :  D. Adam & Associates,

Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ Scotland, U.K.

Tel: +44  (0) 7880 805478

E-Mail:  dougadam@globanet.co.uk     http://www.actel.f9.co.uk

2.	COMPOSITION/ HAZARDOUS INGREDIENTS

Polyester resin containing organic/inorganic pigments in aromatic and aliphatic solvents

3.	HAZARD IDENTIFICATION

This product is flammable flash point  21oC-32oC

4.	FIRST AID MEASURES

Eye contact :	Wash out with copious amounts of water.

If irritation persists seek medical advice.

Skin contact :	Wash off with soap and water. Do not use solvents or thinners.

Ingestion      :  Wash out the mouth with water. Obtain medical advice.

Inhalation     :  Move the exposed person into fresh air. Get prompt medical attention.

5.	FIRE FIGHTING MEASURES

Use Carbon dioxide, Dry Powder, Halogenated Hydrocarbons, Foam. Never use 	water.

Fire will produce dense black smoke from wet product containing hazardous products of 	combustion. Decomposition products may be hazardous to health. Use self contained 	breathing equipment.

6.	ACCIDENTAL RELEASE MEASURES

Spillages should be contained using soil or other adsorbent material to prevent 	contamination of drains and water courses. Waste material from spillages should be 	disposed of in accordance to local regulations. Spilled material may cause surfaces to 	become slippery. Clean-up personnel should use respiratory and/or liquid contact protection.

7.	HANDLING AND STORAGE

Store securely in the original labelled containers in cool, dry, well ventilated conditions away 	from heat sparks and open flames.

8.	EXPOSURE CONTROLS / PERSONAL PROTECTION

Ingredient name		%conc range	Symbol	OES.  	 OES  	 Risk phrases        Notation

(8hrs) 	15mins

Xylene, mixture of isomers  >5%&=<10%	Xn,Xi	100	150	20/21-38-10	Sk OES		N-Butyl Acetate 		  >10%&=<25%		100	200	10		OES

1-Methoxy-2-Propyl Acetate>10%&=<25%		150		10		SUP OES

MIBK			  >2.5%&=<5%	F	  50	100	11		Sk OES

Notes:	LTEL = Long term Exposure Limit - 8 hour time weighted average

STEL = Short  term exposure limit - 10 minute reference period

Notations = Sk - risk of  absorption through skin  Sen - Respiratory sensitiser

OES = Occupational exposure Limit	MEL = Maximum exposure limit

OEL’s are from E40 except where marked SUP which are assigned by the supplier of the substance

Exposure controls : 	Respiratory protection is not normally required but if spraying 					applications are involved appropriate air fed masks should be worn.

Ensure sufficient local exhaust ventilation.

Personal protection :	Respiratory, wear air fed mask when spraying, to prevent inhalation.

Hands, the use of suitable solvent resistant gloves is recommended.

Eyes, safety glasses or goggles, (BS2902).

Skin. Apron, overalls

SAFGLAZA.DOC Page 2 of 2

9.	PHYSICAL AND CHEMICAL PROPERTIES

State			:   Liquid.		Flash point	:  >21oC & < 32oC Abel cup

Colour			:   Pigment		Solubility	:  Not water miscible

Lower explosion limit :   1.1% vol		Density @ 25oC:  1.034gms/cc

Vapour density            :   Heavier than air	VOC content    :   .53 Kgs/lit

10.	STABILITY AND REACTIVITY

This product is stable in excess of  one year when stored between 5OC and 30OC. See sec. 7

11.	TOXICOLOGICAL INFORMATION

There is no data on this product. Exposure to organic solvent vapours may result in adverse 	health effects. Eye splashes and skin care should be avoided so as to prevent irritation.

12.	ECOLOGICAL INFORMATION

No data available. Do not release into surface waters.

13.	DISPOSAL CONSIDERATIONS

Do not release into surface waters. Wastes including empty containers should be disposed of 	in accordance with the Control of Pollution & Environmental Protection Acts.

Disposal of large quantities as waste must always comply with existing EEC,  national and

local regulations. Registered waste carriers and licensed disposal sites must be used.

14.	Transport considerations

Road/Rail	:  SI  number	......1263	pACKING GRP ...111	class...3a

IMDG		:  Packing grp <=30 lts....111	>30lts....111		Class...3.2

Marine pollutant		: Yes	                         UN No 1263

Main risk			: Flammable liquid

Shipping name		: Paint

ICAO/IATA	:  Packing grp <=30 lts....111	>30lts....111		Class...3

UN No 1263

Main risk    			: Flammable liquid

Shipping name		: Paint

INTERNATIONAL: Packing grp		111				Class ...3

ROAD / RAIL	     Main risk			: Flammable liquid	UN No 1263

Shipping name		: Paint

15.	REGULATORY INFORMATION

This product is classified and labelled for supply in accordance with the Chemicals 	(Hazard Information, packaging for supply) Regulations 1994 as follows:

15.1	Classification	:   Flammable

15.1	Labelling	:   Flammable liquid

15.2	Symbol (s)	:

15.3	R-phrases	:   R - 10,  Flammable

15.4	S-phrases	:   S - 2     Keep out of reach of children

S - 7/9  Keep container closed and in a well ventilated place

S - 16,  Keep away from sources of ignition. No smoking.

S - 29   Do not empty into drains

P-phrases	    For use only in commercial and industrial applications

The provisions of the Health & Safety at Work Act and the Control of Substances Hazardous 	to Health Regs apply to the use of this product at work

16.	OTHER INFORMATION

Text of any R phrases in Sec 8:  R10	Flammable

R11	Highly Flammable

R20/21	Harmful by inhalation and in contact with skin

R38	Irritating to skin

This Material Safety Data Sheet dated 20/01/1997 follows the requirements of EEC directive 	91/155/EEC		Revision date May 1999