TECHNICAL DATA SHEET. 			TECHGTI.DOC Page 1 of 2

1.	IDENTIFICATION OF PRODUCT AND COMPANY

1.1	Product name : 	COME CLEAN

Heavy duty organic graffitti remover for hard  surfaces

1.2	Company        :  D. Adam & Associates / ACTEL Coatings

Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ Scotland, U.K.

Tel: +44  (0) 7880 805478  E:  dougadam@globalnet.co.uk    www.actel.f9.co.uk

2.	TECHNICAL DATA

2.1	STORAGE:		Store in a well ventilated area away from sparks and naked

flames.

2.2	APPLICATION: 	For removing graffitti from ARMAGLAZE or from hard

surfaces

2.3	OVERCOATING:	N/A

2.4	REACTION TIME:	10 minutes. May need re-application

2.5	CONSUMPTION:	Typically, 0.1 to 0.3 litres per square metre depending upon

nature 	of graffitti, number of applications and surface being treated.

2.6	PACK SIZE:		4 x 5 ltr carton, 25 litre or 200 litre drums.

2.7	FLASH POINT:	 	38oC

3.	PREPARATION

3.1	Surfaces should be dry before applying COME CLEAN.

3.2	Excessive thick coats of emulsion or gloss graffitti paint should be wire brushed or

scraped to remove as much of the solids build up as possible before applying the product.

3.3	COME CLEAN is also available in a GEL and poultice formulations

3.4	N.B. DO NOT USE ON solvent sensitive, or decorative surfaces without checking fully for any adverse reaction before use.

TECHGTI.DOC Page 2 of 2

4.	APPLICATION

4.1	COME CLEAN as supplied is ready to use.

5.	GRAFFITTI  REMOVAL

5.1	Graffitti damage can be removed as follows:

5.2	Apply COME CLEAN generously by brush or wet rag to graffitti and allow at least 10 minutes reaction time or longer, as indicated by agitating to check the extent of the breaking down of the graffitti.

  - Agitate with a stiff brush to ensure thorough softening of the graffitti.

5.4	Allow  up to a further 1/2  hour reaction time after agitating before wiping the treated

area clean with cloths.

5.5	Repeat the whole process as necessary as soon as the area is dry.

5.6	On untreated surfaces, i.e. surfaces not previously treated with sacrificial coatings, graffitti may penetrate the surface pores to a depth which could make complete removal extremely difficult. Shadows may be removed using Outline as directed.

5.7	After use all treated areas should be rinsed clean by washing with water and a

detergent.

5.8	All equipment should be cleaned with water.

6.	HEALTH AND SAFETY

6.1	Please ensure that operators read and understand the health and safety data sheet before using these materials.

Revised May1999