TECHNICAL DATA SHEET. 			TECHGTI.DOC Page 1 of 2

1.	IDENTIFICATION OF PRODUCT AND COMPANY

1.1	Product name : 	CLEANAWAY GTi

Heavy duty water miscible graffitti remover for porous surfaces

1.2	Company        :  ACTEL Coatings.,

Clarendon, 17 High Cross Avenue,  Melrose TD6 9SQ   Scotland, U.K.

Tel: +44  1786 472465 / 07880 805478

http://www.actel.f9.co.uk    E-Mail: dougadam@globalnet.co.uk

2.	TECHNICAL DATA

2.1	STORAGE:		Store in a well ventilated area away from sparks and naked flames.

2.2	APPLICATION:		For use in removing graffitti from porous and previously untreated 				building surfaces, or for easy removal of graffitti damage from 					surfaces protected by Enviroguard or Envirex.

2.3	OVERCOATING:	N/A

2.4	REACTION TIME:	1 hour up to 8 hours, very dependent on weather conditions.

2.5	CONSUMPTION:	Typically, 0.1 to 0.5 litres per square metre depending upon nature 				of graffitti, number of applications and surface being treated.

2.6	PACK SIZE:		25 litre or 200 litre drums.

2.7	FLASH POINT:	 	90oC

TECHGTI.DOC Page 2 of 2

3.	PREPARATION

3.1	Surfaces should be dry before applying Cleanaway GTi.

3.2	Excessive thick coats of emulsion or gloss graffitti paint should be wire brushed or scraped to 	remove as much of the solids build up as possible before applying the product.

3.3	Cleanaway GTi is also available in a poultice formulation for stubborn stains or for removal 	of old latex flooring adhesives from stone or concrete floors.

3.4	N.B. On solvent sensitive, or decorative surfaces check for any adverse reaction before use.

4.	APPLICATION

4.1	CLEANAWAY GTi as supplied is ready to use.

5.	GRAFFITTI  REMOVAL

5.1	Graffitti damage can be removed as follows:

5.2	Apply Cleanaway GTi generously by brush to graffitti and allow at least one hour reaction 	time or longer, as indicated by the breaking down of the graffitti.

5.3	Agitate with a stiff brush to ensure thorough softening of the graffitti. On stone or brick,  use 	of a brass wire brush will prevent surface scratching and facilitate the break up of graffitti.

5.4	Allow a further 1/2 to 1 hour reaction time after agitating before washing the treated area 	clean with a water pressure wash, preferably hot water.

5.5	Repeat the whole process as necessary as soon as the area is dry.

5.6	On untreated surfaces, i.e. surfaces not previously treated with ACTEL sacrificial 	coatings, graffitti may penetrate the surface pores to a depth which could make complete 	removal extremely difficult. Some shadows may be removed using Outline as directed.

5.7	After use all treated areas should be rinsed clean by washing with water.

5.8	All equipment should be cleaned with water.

6.	HEALTH AND SAFETY

6.1	These products are considered to be non hazardous. However, please ensure that operators 	read and understand the health and safety data sheet before using these materials.

Revised May 1999