TECHNICAL DATA SHEET. 		TECHOUTLDOC Page 1  of 2

1.	IDENTIFICATION OF PRODUCT AND COMPANY

1.1	Product name : 	OUTLINE

Water based graffitti “ghost” remover

1.2	Company        :  D. Adam & Associates / ACTEL Coatings

Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ Scotland, U.K.

Tel: +44  (0) 7880 805478  E:  dougadam@globalnet.co.uk    www.actel.f9.co.uk

2.	TECHNICAL DATA

2.1	STORAGE:		Store in original labelled containers in a secure, well ventilated area.

Keep out of reach of children.

2.2	APPLICATION:		Use for removal of residual marker pen graffitti “shadows” which

may remain after the surface has been initially cleaned.

2.3	OVERCOATING:	N/A

2.4	REACTION TIME:	At least 6 hours, preferably overnight.

2.5	CONSUMPTION:	0.1 to 0.5 litres per square metre depending upon nature of graffitti,

number of applications and surface being treated

2.6	PACK SIZE:		25 litre or 200 litre drums.

2.7	FLASH POINT:	 	N/A

TECHOUTL.DOC Page 2  of 2

3.	PREPARATION

3.1	Remove as much graffitti damage as possible using Cleanaway GTi.

(This is an important step in the process since OUTLINE reacts 	symbiotically with

the GTi pre treatment).

3.2	After cleaning with GTi wash the surface with water and ensure that the 				surface is damp before applying OUTLINE

3.3	N.B. On solvent sensitive, or decorative surfaces check for any adverse reaction before use.

4.	APPLICATION

4.1	OUTLINE as supplied is ready to use.

4.2	Brush OUTLINE generously over the residual shadows on the previously prepared surfaces.

4.3	Leave the OUTLINE in contact for at least 2 - 6 hours or preferably overnight.

4.4	After the prescribed contact time ensure that the treated area is washed using copious 	amounts of clean water.

4.5	All equipment should be cleaned with water after use.

5.	GRAFFITTI  REMOVAL

5.1	Graffitti shadows can be removed as follows:

5.2	First remove the bulk of the surface graffitti using Cleanaway GTi in accordance with the 	technical data sheet for that product.

5.3	Wash the treated area clean with water .

5.4	Remaining shadows may be removed using ACTEL OUTLINE.

5.5	Apply OUTLINE generously by brush to the shadows while the area is still wet from

the initial graffitti removal process. Leave in contact for at least 6 hours or overnight

until the shadowing has disappeared.

5.6	Between applications and on completion of the job all treated areas should be rinsed clean 	by washing with water to remove all product residues.

6.	HEALTH AND SAFETY

6.1	This product is considered to be an irritant to the skin and eyes. Please ensure that operators 	read and understand the health and safety data sheet before using these materials.

Revised May 1999