TECHNICAL DATA SHEET. 			techresort.DOC Page 1 of 2

1.	IDENTIFICATION OF PRODUCT AND COMPANY

1.1	Product name : 	LAST RESORT

Heavy duty paint stripper & graffiti remover

1.2	Company        :  D. Adam & Associates / ACTEL Coatings

Clarendon, 17 High Cross Avenue, Melrose TD6 9SQ Scotland, U.K.

Tel: +44  (0) 7880 805478  E:  dougadam@globalnet.co.uk    www.actel.f9.co.uk2.	TECHNICAL DATA

2.1	STORAGE:		Store in a well ventilated area away from sparks and naked

flames.

2.2	APPLICATION: 	For removing paint and graffitti from buildings surfaces and

steelwork

2.3	OVERCOATING:	N/A

2.4	REACTION TIME:	10 minutes. May need re-application

2.5	CONSUMPTION:	Typically, 0.1 to 0.3 litres per square metre depending upon

nature 	of graffitti, number of applications and surface being treated.

2.6	PACK SIZE:		4 x 5 ltr carton and  25 litre drums.

2.7	FLASH POINT:	 	n/a

3.	PREPARATION

3.1	Surfaces should be dry before applying LAST RESORT.

3.2	Excessive thick coats of emulsion or gloss graffitti paint should be wire brushed or

scraped to remove as much of the solids build up as possible before applying the product.

3.3	N.B. DO NOT USE ON solvent sensitive, or decorative surfaces without checking fully for any adverse reaction before use.

Techresort.doc Page 2 of 2

4.	APPLICATION

4.1	LAST RESORT as supplied is ready to use.

5.	GRAFFITTI  REMOVAL

5.1	Graffitti damage can be removed as follows:

5.2	Apply LAST RESORT generously by brush or wet rag to graffitti and allow at least 10 minutes reaction time or longer, as indicated by agitating to check the extent of the breaking down of the graffitti.

  - Agitate with a stiff brush to ensure thorough softening of the graffitti. Keep surface wet with LAST RESORT.

5.4	Allow up to a further 1/2  hour reaction time after agitating and then wash down the

the cleaned area preferably with a hot water washer or steam cleaner.

5.5	Repeat the whole process as necessary as soon as the area is dry.

  - On porous brick and stone, concrete surfaces etc. graffitti may penetrate the to a

depth which could make complete removal extremely difficult. Shadows may be

removed using Outline as directed.

5.7       Protect work area to prevent waste water coming into contact with grass and

other vegetation or accessing drains and water channels. Use a wet vac or an absorbent material to collect waste to prevent it gaining access to surface water drains.

5.8	All equipment should be cleaned with water.

6.	HEALTH AND SAFETY

  - Please ensure that operators read and understand the health and safety data sheet

before using these LAST RESORT.

  - Do not use LAST RESORT in an enclosed area without very adequate ventilation.

6.3     Wear suitable clothing to protect the hair, eyes and skin when using LAST RESORT

Revised May1999